Scope

Applicable to maintenance and repair work on installations and machines

What and how?

- Interruption of the entire power supply to the installation:

Make sure that the installation to be worked on is fully switched off.

Switch the switch to the off position (position zero).

Consider all forms of energy (electricity, steam, hydraulic liquid under pressure, compressed air, etc.).  The interruption has to be done in a visible way

- Precautions against switching back on:

Make sure that the installation cannot be switched on again without permission.

Four possible ways:

- Message (tag) to hang up at switch or closing device with the order to not switch back on

- Removal of safety fusing or  locking the safety fusing with a lock mechanism

- Fitting a padlock on the switching device

- Disconnection of pressure pipes

- Neutralisation of energy:

Any residual energy that can still be present in the installation must be neutralised.

This particularly concerns energy that can have accumulated in:

- Electrical capacitors

- Still moving parts (inertia)

- Compressed gases

- Masses hoisted at height (to be lowered or blocked, mechanically or with structural member)

- Control

The control is to check that there are no defects and no human errors have been made (such as operating a wrong switch).  Carry out the control with all precautions one would take if energy was still present in the installation.

KUWAIT PETROLEUM NORTH WEST EUROPE

DOCUMENT TITLE:	Lock out tag out of machines

DOCUMENT NUMBER:	KPNWE.WI.11.HSCO.019

REVIEW NUMBER: 	1

EFFECTIVE DATE:    	17 Jul 2020

NEXT REVIEW DATE:	17 Jul 2023

..

CONFIDENTIALITY:

The information contained in this document is confidential to Kuwait Petroleum International Ltd. Copyright © Kuwait Petroleum International Ltd.  Copying of this document in any format is not permitted without written permission from the management of Kuwait Petroleum International Ltd.

This document is reviewed and approved according to the released online Document Approval Flow

| Prepared by: | Operational Assistant | An Cornelis |
| --- | --- | --- |
| Reviewed by: | Operational Assistant | An Cornelis |
| Approved by: | QHSSE Manager | Gerardus Timmers |

# *Access rights:	       Generally Accessible

Reviews Summary

| Review date | Review reference | Review details | Review version |
| --- | --- | --- | --- |
| 01 Dec 2017 |  |  | 0 |
| 17 Jul 2020 | -- | minor changes | 1 |