- INTRODUCTION TO MEASUREMENTS

The term "Measuring is knowing and guessing is missing" speaks for itself. When measurements are required for a specific task, i.e. working in hazardous atmospheres or where the work itself can create it, it is important that this is done in a correct manner with the appropriate tools.

| - REQUIREMENTS FOR MEASUREMENTS |  |
| --- | --- |
| CHECK |  |
| - GENERAL GUIDELINES FOR MEASURING EQUIPMENT |  |
| - Only authorised, calibrated and inspected measuring equipment |  |
| - valid and identified by law/governing body |  |
| - Only trained persons |  |
| - are permitted to take measurements and observations must be properly interpreted |  |
| - Inspect the equipment before each measurement |  |
| - WHICH MEASUREMENTS |  |
| - Oxygen content |  |
| - with oxygen meter, - minimum permissible oxygen concentration: 18 % (or more stringent according to local legal requirements) |  |
| E.g.: Entry into tanks after degassing, when welding in a confined space, ... |  |
| - Vapour or gas concentrates |  |
| - with explosion meter - maximum permissible concentration: 10 % of the LEL |  |
| E.g.: when working in explosive areas in depots, inside pumps in stations, ... |  |
| - Toxic substances |  |
| - with gas-identification tubes or special meters - maximum permissible concentration: - general 50 % of the MAC value (maximum allowable concentration) - for CO 10 ppm (parts per million) |  |
| E.g.: When welding in confined spaces, excavation of contaminated soil ... |  |
| - FREQUENCY OF MEASUREMENTS |  |
| - Always BEFORE START of work / entry / access |  |
| - CONTINUOUS, if |  |
| - dust is released while working - there is no certainty about the presence of substances |  |

KUWAIT PETROLEUM NORTH WEST EUROPE

DOCUMENT TITLE:	TSR - Measurements / Hazardous atmospheres

DOCUMENT NUMBER:	KPNWE.WI.11.HSCO.075

REVIEW NUMBER: 	2

EFFECTIVE DATE:    	08 Jul 2020

NEXT REVIEW DATE:	08 Jul 2023

..

CONFIDENTIALITY:

The information contained in this document is confidential to Kuwait Petroleum International Ltd. Copyright © Kuwait Petroleum International Ltd.  Copying of this document in any format is not permitted without written permission from the management of Kuwait Petroleum International Ltd.

This document is reviewed and approved according to the released online Document Approval Flow

| Prepared by: | HSSE Supervisor | Jan Berghmans |
| --- | --- | --- |
| Reviewed by: | HSSE Supervisor | Jan Berghmans |
| Approved by: | QHSSE Manager | Gerardus Timmers |

# *Access rights:	       Generally Accessible

Reviews Summary

| Review date | Review reference | Review details | Review version |
| --- | --- | --- | --- |
| 01 Dec 2017 |  |  | 0 |
| 15 Feb 2018 | -- | Doc title changed from 'TSR - Measurements of dangerous goods' to 'TSR - Measurements / Hazardous atmospheres' | 1 |
| 08 Jul 2020 | -- | minor changes | 2 |